NSTX Upgrade

Centerstack Casing and Lower Skirt Stress Summary

NSTXU-CALC-133-03-00

Rev 0

February 10, 2012

Prepared By:

___________________________________

Peter Titus, PPPL Engineering Analysis Branch,

Reviewed By:

_____________________________

A.Brooks

______________________________

A.Zolfaghari

______________________________

Irving Zatz (For Jim Chrzanowski),

NSTX Cognizant Engineer

PPPL Calculation Form

Calculation # 	NSTXU-CALC-133-03-00    Revision #  00		WP #, 0029,0037

(ENG-032)

Purpose of Calculation: (Define why the calculation is being performed.)

The purpose of this calculation is to qualify the new centerstack casing and lower support skirt for the Upgrade loads.   The stress contributions are generated in this calculation or gathered from other calculations.  A stress summation and summary for the centerstack casing is presented and compared with allowables.

References (List any source of design information including computer program titles and revision levels.)

-See the reference list in the body of the calculation

Assumptions (Identify all assumptions made as part of this calculation.)

Referenced calculations include assumptions.

Calculation (Calculation is either documented here or attached)

Attached in the body of the calculation

Conclusion (Specify whether or not the purpose of the calculation was accomplished.)

Stress levels are below the static and fatigue allowables for the Inconel 625 shell. High strength bolts are needed.  ASTM A193 B8M class 2 bolts are recommended for all flange bolts, with appropriate preloading (torqued to 75% yield for conventional through bolts - less for the blind tapped holes in the PF1b mandrel).  Welds should be the full thickness of the thinner shell segment. The PF1b mandrel is in the main load path that transfers the casing loads to the support skirt. In the fall of 2011, there were no spacers between flanges connected by the studs, and the studs would not load in compression. The studs will not contribute adequately to the moments and compression loads at the base of the casing. It is recommended that tubular spacers be added to the studs.  Bolt stresses at the lower casing flange and skirt flanges are significantly stressed (60 to 70 ksi). These should be preloaded high strength bolts. These have an allowable of 62 ksi, so, they are slightly undersized. Peak stress is at the coax opening of the skirt. Increasing the bolt size on either side of the opening should be considered.  The shallow thread blind tapped bolt connection of the PF1b mandrel connection to the PF1a mandrel flange doesn't have adequate capacity to manage the last round of Halo loads. Doubling the number of holes is recommended. High strength bolts are recommended in the "softer" 316 flange - this allows a bit higher shear based on the Federal Screw Fasteners Standard. They will also have to be preloaded to take the lateral load in friction and to develop a greater moment carrying capacity. Welding on the high strength bolt threads would degrade their capacity. Use of Locktite is recommended.

Loading from the CHI electrical connections have been included in the assessment of the net loads on the casing, but the stresses in the CHI rod and the supports for this rod and the reactions from the bus bar connections have not yet been analyzed because details of theses supports are lacking.

Cognizant Engineer’s printed name, signature, and date

Irving Zatz (for Jim Chrzanowski)

I have reviewed this calculation and, to my professional satisfaction, it is properly performed and correct.

Checker’s printed name, signature, and date:

_____________________________________________________A. Brooks

_____________________________________________________A. Zolfaghari

# 2.0 Table of Contents

Centerstack Casing Stress Summary

Section.Paragraph

Title Page                          	         							1.0

ENG-033 Form									1.1

Table Of Contents                            							2.0

Executive Summary								3.0

Digital Coil Protection System Input						4.0

Design Input, 									5.0

Criteria                             							5.1

Design Point Spreadsheet Loads						5.2

References                                   						5.3

Material Properties and Allowables						5.4

Photos and Drawings of Components					5.5

Analysis Models									6.0

Global Model 								6.1

Pressure Loading									7.0

Normal Operating Vacuum Loading 					7.1

Test Vacuum Loading and Buckling					7.2

Heat Balance Results 								8.0

Halo Current Results								9.0

Halo Currents In the Casing						9.1

Halo Currents in the CHI Bus Connection					9.2

Inductive Eddy Current Loads

Mid Plane Disruption, Quench of P1					10.0

VDE’s									11.0

Slow Mid Plane Translation and Quench P1 to P2				11.1

P1 to P5 10 ms VDE Fast Quench at P5					11.2

Tile Disruption Loads								12.0

PF Loading Results								13.0

Stress Results from Ref [2]						13.1

Buckling								13.2

Torsional Loading from TF OOP loads						14.0

Seismic Loading Results								15.0

Skirt Stresses									16.0

Lower Casing and Skirt Bolt Stress							17.0

Upper  PF1b Flange Bolting and Weld					17.1

Lower PF1b Flange Bolting						17.2

Skirt Upper Flange Bolting						17.3

Skirt Lower Flange Bolting						17.4

Bolt Loads on the Lower Flag Keys						17.5

Appendix A - Email Correspondence On loads, requirements etc.

Appendix B  - Ref [19] Halo reaction loads for the base skirt  with the compliance of the G-10 flange modeled.

3.0 Executive Summary

Figure 3.0-1 PDR Qualification of Casing Elements

This is a collection of results from other calculations intended to assess the total stress in the centerstack casing. Some stress calculations have been added beyond the references to complete assessment of the load inventory. Figure 3.0-1 shows the PDR status of the qualification of some of the elements of the casing structure. All of these components and loads have been re-visited in the final design.

The first component of normal operating stress comes from the inner PF analyses [2]. The inner PF coils, PF 1a and b upper are supported by the casing. Net vertical loads for the upper coils PF1a and b may be found in the Design Point spreadsheet [1], load combinations sheet. These are also included in section 5.2 of this calculation. Reference [2] calculates these independently from the 96 equilibria. The TF coils indirectly load the casing as well because the casing is one of the redundant or statically indeterminant load paths that resist the TF out-of-plane loads. The torsional shear stresses in the casing are quantified in the global model calculation, ref [9] and are summarized in section 14.0.  During a normal shot, the heat load on the tiles heats up the casing, but there is active cooling at the flanged ends of the casing to protect the Viton seals and PF1b which is very close to the flange. The thermal gradients in the casing and the conical sections of the casing cause stresses that will superimpose on the PF Lorentz load stresses. The heat balance calculation, reference [3], computes the heat transfer throughout the interior of the vessel from plasma heating of tiles and exposed sections of the vessel. Heat is conducted through the centerstack tiles and the inner divertor and reference [3] quantifies the casing temperature. A stress pass is included in the analysis and provides the stress to be added to other loading components.

Figure 3.0-2 Stress due to Halo Currents

Disruption loads are addressed in reference [4] for halo currents and reference [10] for the inductively driven axisymmetric stresses in the casing wall. Inductive current stresses are less than 50 MPa. The halo current loads represent a potentially complex set of loads that depend on the entry and exit points, described in the GRD.  They also depend on loading time durations that preclude resistive re-distribution of the non-axisymmetric halo currents for very fast disruptions, and allow resistive re-distribution for slow disruptions. The non-axisymmetric loading that results from the fast disruptions loads the casing dynamically and is addressed by a transient structural calculation. The casing inertia and the spring restraint provided by the bellows limits the stress in the casing. Tile weights used for the inertial components of the model come from the tile stress calculation, ref [6], and the bellows analysis that provided the stiffness, reference [5]. Loads at the bellows spring are a part of the bellows loading addressed in the bellows stress calculation, reference [5]. The bellows spring rate and the cantilever stiffness of the centerstack is an important component of the magnetic stability analysis performed in reference [7]. The centerstack casing is vertically cantilevered from the pedestal. Stresses due to seismic overturning loads may be found in reference [12].

Figure 3.0-3 FDR Presentation Showing Two of the Many Load Components on the Centerstack Casing.

Table 3.0-1 Centerstack Casing Stress Summary (stresses in MPa)

For the FDR, an envelope of the stresses was presented and the sum was reported as 414 MPa, which was close to the fatigue allowable for Inconel 625.  In table 3.0-1, the stresses are summed at four locations and the peak total is 299.3 MPa. This occurs at a weld in the casing, shown in Figure 5.5-3. The weld is a full penetration through the thinner section and backed with a 1/6th seal weld. The stress calculations capture the stress in the full penetration weld. As long as this meets acceptance criteria, the 1/16” weld is redundant.

The possibility of buckling the 1/4 inch casing shell is addressed in ref [2] and additionally in section 13.2 of this calculation.

Bolts at the lower casing flange and skirt flanges are significantly stressed (60 to 70 ksi). These should be preloaded high strength bolts. ASTM A193 B8M class 2 bolts are recommended. These have an allowable of 62.5 ksi, so, they are slightly undersized. Peak stress is at the coax opening of the skirt. Increasing the bolt size on either side of the opening should be considered.

The PF1b mandrel is in the main load path that transfers the casing loads to the support skirt. Currently, there are no spacers between flanges connected by the studs, and the studs will not load in compression. The studs will not contribute adequately to the moments and compression loads at the base of the casing. It is recommended that tubular spacers be added to the studs. Ref [7] calculates the stress due to an offset between the magnetic and structural centers due to tolerances. From [7], the stress due to the manufacturing tolerance would be a maximum of 0.43 MPa and the bellows stress would be 9.77 MPa.

Recent questions regarding the halo current loading on the centerstack casing highlights a design weakness in the support of the casing. The upper bellows provides minimal lateral restraint, and the casing is basically cantilevered from the lower structures and bolt circles. Halo Loads and, particularly, moments were acceptable when credit was taken for mitigation of the peaking factor from dynamic effects and resistive redistribution of the asymmetric currents. From October 2011 emails, there is a large uncertainty in the halo loads - Art Brooks suggests enveloping the uncertainty by assuming the worst loading at the mid plane  of 50,000 lbs. This would produce a moment of 50000*(1.6m*39.37+22in) = 4.2e6 in-lbs. The skirt bolt pattern has a section modulus of 19 in^3 and the bolt stress would be 237,000 psi. The bolts to the g-10 ring, and to the inserts in the TF flags, and connection through the lower crown to the pedestal, would also see high loads.

It was recommended that a lateral restraint be added at the upper bellows elevation - a slip ring or struts. They would have to take half the 50000 lb halo load. This would much reduce the moment at the base and add needed margin against loading that  probably won't be able to be quantified until the upgrade has operated. Art Brooks recalculated the reaction loads and moments at the base (Appendix B).

Updated Halo loading of the lower G-10 ring and it's connections to the TF flags, is evaluated in ref [20].

4.0 Digital Coil Protection System (DCPS) Input

Casing Stress:

Most of the loading on the casing is either thermal or disruption loading. The DCPS typically is concerned mainly with coil Lorentz force derived stresses. Table 3.0-1 lists the Lorentz Force derived stress as 45 MPa. It occurs at the intersection of the straight section and flare. This comes from L. Myatt's calculation of the casing stresses from the inner PF coils, ref [2]. The 45 MPa will scale based on the net vertical load from PF1a and b upper. Myatt used the worst of the 96 scenarios, which corresponds to the 67939 lbs from the design point spreadsheet - excerpt at right. The DCPS should compute the casing Lorentz Stress from:

(Sum of PF1a and b Vertical loading in lbs ) * 45MPa /67939lbs = Lorentz Stress

The max stress in the casing is 200 MPa for 96 equilibria, plus thermal and disruption loads. With the Lorentz portion of the stress at 45 MPa, the "headroom” needed for Non-Lorentz Loads is 155 MPa.

The static allowable is 450 MPa so the Lorentz stress could go to 300 MPa, and still pass the static allowable. The worst case Max load is 257587lbs - this would produce a casing stress of 257587/67939*45 = 170 MPa - so there is only marginally a possibility that currents in their worst configuration could cause an  unacceptable stress - but the bolting in the lower flange will fail before this stress could be reached.

Lower Casing Support Bolts

Because they are sized to the worst halo loads, there isn't much margin to take anything more than the total PF 1a,b upper and lower Lorentz launching load that was used in section 17 to qualify the bolts. This is 25161 lbs from Table 5.2-1. Maintaining the net PF1a,b upper and lower summation below this value will protect the bolting from halo loads during a disruption. If more margin is needed to allow a better operating window, the halo loads on the bolts will have to be re-visited.

5.0 Design Input,

5.1 Criteria

Criteria may be found in reference [8],   NSTX Structural Design Criteria Document, I. Zatz.

5.2 Design Point Spreadsheet Loads

Reference [2] addresses the stress in the centerstack due to the loads from PF1a and b upper. The bolting at the lower end of the casing assembly is exposed to the net loads from PF1a, and b, upper and lower. This summation is available in the Design Point spreadsheet, reference [1].

Table 5.2-1 Net Vertical Loads on the Lower Connections of the Skirt

Table 5.2-2 Net Vertical Loads on the Lower Connections of the Skirt Plus OH Coil

| Fz(lbf) | (PF1AU+PF1BU+PF1BL+PF1AL+OH) |
| --- | --- |
| Min w/o Plasma | -39635 |
| Min w/Plasma | -53445 |
| Min Post-Disrupt | -41843 |
| Min | -53445 |
| Worst Case Min | -375500 |
| Max w/o Plasma | 20397 |
| Max w/Plasma | 10748 |
| Max Post-Disrupt | 19630 |
| Max | 20397 |
| Worst Case Max | 375501 |

5.3 References

[1]  http://www.pppl.gov/~neumeyer/NSTX_CSU/Design_Point.html  Dated 2 -17- 2009

[2] WBS 1.1.3 Structural Analysis of the PF1 Coils Leads and Supports, Rev1 NSTX-CALC-133-01-01

Prepared By: Leonard Myatt, Reviewed by: TBD, Cognizant Engineer: Jim Chrzanowski

[3] WBS 1.1.1 Plasma Facing Components,  Global Thermal Analysis of Center Stack – Heat Balance NSTX-CALC-11-01-00  Prepared By: Art Brooks, Reviewed by: Han Zhang, Cognizant Engineer: Jim Chrzanowski

[4] WBS 1.1.3 Magnet Systems, Halo Current Analysis of Center Stack  NSTXU-CALC-133-05-00  Prepared By: Art Brooks, Reviewed by: Peter Titus, Cognizant Engineer: Jim Chrzanowski

[5] Bellows Qualification Calc # NSTXU CALC 133-10-00, Peter Rogoff’

[6] Tile Stress Analysis (ATJ) NSTXU CALC 11-03-00, Art Brooks  Used to include tile weights into the effective density of the centerstack casing, transmitted via email:

Peter, Pete: Attached are the volumes Ankita extracted from the ProE models. The density of the Center Case (inconel) is 8440 kg/m3, the tile (ATJ Graphite - www.graftech.com) is 1760 kg/m3 giving a total mass of 1138 kg and an effective density if the CS (which includes the mass of the tiles) of 12,248 kg/m3.

[7] OH & PF1 & 2 Electromagnetic Stability Analyses NSTXU-CALC-133-11-00 Rev 0 March 2 2010  Prepared By: Peter Titus, , Reviewed By: Ali Zolfaghari Cognizant Engineer: Jim Chrzanowski WBS 1.1.3

[8] NSTX Structural Design Criteria Document, NSTX_DesCrit_IZ_080103.doc I. Zatz

[9] NSTX-CALC-13-001-00 Rev 1  Global Model – Model Description, Mesh Generation, Results, Peter H. Titus  June  2011

[10] WBS 1.1.1 Disruption Analysis of Passive Plates, Vacuum Vessel & Components

NSTXU-CALC-12-01-01 Rev 1 April, 2011 Prepared By: Peter Titus, Contributing Authors: A. Brooks, Srinivas Avasarala,  J. Boales Reviewed By: Yu Hu Zhai, Cognizant Engineer: Peter Titus

[11] National Spherical Torus Experiment NSTX CENTER STACK UPGRADE GENERAL REQUIREMENTS DOCUMENT NSTX_CSU-RQMTS-GRD Revision 0 March 30, 2009 Prepared By: Charles Neumeyer NSTX Project Engineering Manager

[12] NSTX Upgrade Seismic Analysis NSTXU-CALC-10-02-00 Rev 0 February 9  2011 Prepared By: Peter Titus, reviewed by F. Dahlgren

[13] Analysis of the NSTX Upgrade Centerstack Support Pedestal  NSTXU-CALC-12-09-00 May 2011 WBS 1.1.2 Prepared By: Peter Titus  Reviewed By: Ali Zolfaghari, Cognizant Engineer: Mark Smith

[14] MODELLING OF THE TOROIDAL ASYMMETRY OF POLOIDAL HALO CURRENTS

IN CONDUCTING STRUCTURES N. POMPHREY, J.M. BIALEK_, W. PARK, Princeton Plasma Physics Laboratory,Princeton University,  Princeton, New Jersey,

[15] Email from Art Brooks Thu 3/11/2010 8:21 AM, providing Upper and Lower design loads for the centerstack casing halo loads, copy of the email is included in the appendices

[16] NSTX Basic Tile Analysis Qualification

NSTX-CALC-11-02-00 December  2010 Prepared By: J. Boales

[17] "General Electric Design and Manufacture of a Test Coil for the LCP", 8th Symposium on Engineering Problems of Fusion Research, Vol III, Nov 1979

[18] "Handbook on Materials for Superconducting Machinery"  MCIC- HB-04  Metals and Ceramics Information Center, Battelle Columbus Laboratories 505 King Avenue Columbus Ohio 43201

[19] email from Art Brooks November 1 2011, providing Halo reaction loads for the base skirt  with the compliance of the G-10 flange modeled.

[20] STRUCTURAL CALCULATION OF THE TF FLAG KEY,Rev 1 NSTXU-CALC-132-08-01  December 05, 2011 Prepared By: Ali Zolfaghari

[21] Progress on CHI and MGI Experiments on NSTX R. Raman, 53rd Meeting of the Division of Plasma Physics, APS 2011 Conference November 14-18 2011 Salt Lake City

[22] FED-STD-H28 Federal Standard Screw Thread Standards for Federal Services FED-STD-H28 March 31 1978

[23] NSTX Upgrade OH Coaxial Cable and Embedded LeadsNSTXU-CALC-133-07-00  September 7 2011 Prepared By: Michael Mardenfeld, PPPL Mechanical Engineering

5.4 Material Properties and Allowables

Table 5.4-1 Tensile Properties  for Stainless Steels

| Material | Yield, 292 deg K (MPa) | Ultimate, 292 deg K (MPa) |
| --- | --- | --- |
| 316 LN SST | 275.8[17] | 613[17] |
| 316 LN SST Weld | 324[17] (23.3ksi) | 482[17] 553[17] |
| 316 SST Sheet Annealed | 275[18] | 596[18] |
| 316 SST Plate Annealed |  | 579 |
| 304 Stainless Steel (Bar,annealed) | 234 33.6ksi | 640 93ksi |
| 304 SST 50% CW | 1089 | 1241 180ksi |

Table 5.4-2 Coil Structure Room Temperature (292 K) Maximum Allowable Stresses, Sm = lesser of 1/3 ultimate or 2/3 yield, and bending allowable=1.5*Sm

| Material | Sm | 1.5Sm |
| --- | --- | --- |
| 316 Stainless Steel | 184 MPa, 26.7 ksi | 276 MPa |
| 316 Weld | 161 MPa | 241 MPa |
| 304 Stainless Steel (Bar,annealed) | 156MPa(22.6ksi) | 234 MPa (33.9ksi) |

Figure 5.4-1 Weld Allowable

Inconel 625 properties are shown below and in Figure 5.4.2-. Fatigue allowables from ref [2] are shown in figure 5.4-3

## ASTM A193 Bolt Specs from PortlandBolt.com

| B8 | Class 2 Stainless steel, AISI 304, carbide solution treated, strain hardened |
| --- | --- |

## Mechanical Properties

| Grade | Size | Tensile ksi, min | Yield, ksi, min | Elong, %, min | RA % min |
| --- | --- | --- | --- | --- | --- |
| B8 Class 2 | Up to 3/4 | 125 | 100 | 12 | 35 |
|  | 7/8 - 1 | 115 | 80 | 15 | 35 |
|  | 1-1/8 - 1-1/4 | 105 | 65 | 20 | 35 |
|  | 1-3/8 - 1-1/2 | 100 | 50 | 28 | 45 |

The ASTM A193 B8M Class 2 5/8 inch Bolts would have a Stress Allowable of the lesser of 125/2 or 2/3*100 =62.5 ksi

From Ref 2

Center stack coil support structure is made from Inconel 625:

Sy~65 ksi, Sut~130 ksi

Sm~43 ksi (300 MPa)

Membrane + Bending Stress Limit at RT: (1.5)300=450 MPa

Max Cyclic Stress (58.5k cycles) = 375 MPa (R~0.05)

Figure 5.4-2 Inconel 625 Properties

Figure 5.4-3 Inconel 625 Fatigue Properties

5.5 Photos and Drawings of Components

Figure 5.5-1

Figure 5.5-2 Upper and Lower Cylindrical Sections

Figure 5.5-3 Casing Dimensions and Weld Details

Figure 5.5-4  Lower Mandrel Assembly

Figure 5.5-5 Lower PFbl Mandrel Assembly

Figure 5.5-6 Lower Mandrel Assembly (October 2011 version with 11 5/8 inch bolts in the lower flange)

Figure 5.5-7 Lower Support Skirt (FDR vintage with 24 1/2 inch bolts at the lower flange)

Figure 5.5-8 Upgrade Centerstack Casing Drawing

6.0 Analysis Models

The referenced calculations include a number of separate models for thermal and electromagnetic modeling. The centerstack casing is included in the global calculation, ref[9].

Figure 6.0-1 Global Model Representation of the Centerstack Casing

Figure 6.0-2 Swept Mesh Model of the Centerstack Casing

Figure 6.0-3 Swept Mesh Model of the Centerstack Casing Showing Displacement Constraints

7.0 Pressure Loading

7.1 Normal Operating Vacuum Loading

Normal operating pressures on the casing come from atmospheric pressure on the inside of the casing and produces hoop tension. The Tresca stress from this loading is 6.3 MPa.

Figure 7.1.1 Operating Vacuum Load Pressure Surfaces, Hoop, and Tresca Stress

7.2 Test Vacuum Loading and Buckling

During final manufacturing tests, leak tests may be performed by closing off the top and bottom flanges and drawing a vacuum on the casing. Helium is "sprayed" on the outside welds and a mass spectrometer is used at the vacuum pumping duct. The casing must be stable with an atmosphere of external pressure. This is the opposite loading experienced during normal operation.

Figure 7.2-1 Test Vacuum Load Pressure Surfaces, Displacements, and Stress

Shell stresses are small during the test vacuum.

Figure 7.2-2 Buckling Modes 1 and 2

The margin against buckling for this eigenvalue buckling calculation is 25.8; well beyond the factor of 5 required in the NSTX Structural Criteria Document.

Figure 7.2-3 Buckling Modes 3 and 5

The first two modes are the most critical. Beyond this the buckling margin goes up beyond 33.

8.0 Heat Balance Results

These results are reproduced from Art Brooks’ heat balance calculation, ref [3].

Figure 8.0-1 Temperature Distribution from the Heat Balance Calculation [3]

Figure 8.0-2 Temperature Stress Distribution from the Heat Balance Calculation [3]

9.0  Halo Current Results

9.1 Halo Currents in the Casing

These results are reproduced from Art Brooks’ Vessel and Internals Heat Balance Calculation, ref [4].

Figure 9.1-1 Halo Disruption Stress Results from [4], presented at the FDR

Figure 9.1-2 Halo Disruption Stress Results from [4]

9.2  Normal Operation and Halo Currents in the CHI Bus Connection

Currents flow in the CHI system during start-up and during a disruption. Normal operation for the upgrade is expected to utilize 27 kA of current [21] when the TF is at full field. This is planned to produce 1 MA of plasma current. This occurs during start-up. The CHI can be used for current drive after the initiation. This was done early in the NSTX program to demonstrate current drive but is not commonly used.

Figure 9.2-1 Normal Operational Parameters Expected for NSTX Upgrade [21]

Figure 9.2-2 Layouts and Arrangement of the CHI Connections

|  |  | NSTX BASE | NSTX CSU |
| --- | --- | --- | --- |
| Ro | m | 0.854 | 0.934 |
| A_100 |  | 1.3 | 1.5 |
| Ip | MA | 1.0 | 2.0 |
| Bt@Ro | T | 0.6 | 1.0 |
| I =5e6*radius*Bt at Radius | Amp | 2.562e6 | 4.67e6 |
| I per Turn = | Amp | 71166 | 129722 |

The toroidal field at the CHI Rod is .934*1.0/(18.31/39.36) = 2.008T.  Normal operating current in the CHI rods is 27 kA/3 = 9kA. The load in the rod over almost a meter of height is 9000*2T = 18000N or 2023 lbs per rod, radially outward.  The Halo loading is 7% of 2e6 amps*2T*1m/3 = 93333N = 20981 lbs per rod. One important observation is that the disruption considered in Art Brooks’ simulation is a centered disruption. The disruption that drives currents in the CHI bus is a quench after a VDE.  So, the loading in the CHI bus is not additive to those loads calculated by A. Brooks.

10.0 Mid Plane Disruption, Quench of P1

This was thought to be an interesting disruption case. At the mid plane, the centerstack casing is not reinforced. The only structural strength comes from the 1/4 inch shell.

Figure 10.0-1 Max Poloidal Fields for All 96 Equilibria

Figure 10.0-2 Current Density Vectors for the Mid-Plane Disruption

Figure 10.0-3 Lorentz Forces Resulting from Toroidal Currents (Figure 10.0-2) and the Poloidal Field (Figure 10.0-1

Figure 10.0-4 Dynamic Casing Stresses for the Worst Time Point (100.007)in the Disruption. See Time History Plot in Figure 10.0-5

Figure 10.0-5 Dynamic Time History Response of the Casing showing the largest deflection occurring at 100.007 seconds.

11.0 Translations and VDE's

11.1 Slow Mid Plane Translation and Quench P1 to P2

Reference [10] includes the mid plane translation and quench. The results are presented in this section. As for the previous mid-plane disruption, the stresses are small.

Figure 11.1-1 CS Casing Stresses for Translation and Quench

Figure 11.0-2 CS Casing Equatorial Plane Radial Displacement Post 26 Time History

11.2  P1 to P5 10 ms VDE Fast Quench at P5

Appendix I of reference [10] introduces another disruption simulation. Simulation of the passive plate disruptions also includes other structures including the centerstack casing.

Mesh, Including Air      Materials      Solid 97 Type

Figure 11.2-1 Appendix I of Reference 10 Electromagnetic model

Figure 11.2-2 Current Densities, P1-P5, 10ms VDE (ends at Step 8) and Quench at P5

Figure 11.2-2 Tresca Stress, P1-P5, 10ms VDE (ends at step 8) and Quench at P5

This Disruption Simulation produces nothing more than 8.08 MPa.

12.0 Tile Moments

The tile eddy currents will produce moments about the vertical axis and radial  - Individual tile moments applied to an axisymmetric structure

Figure 12.0-1 Tile Inventory from Kelsey Tresemer's FDR Presentation

From Ref [16]:

CSFW Tiles

There are two types of CSFW tiles: fixed tiles and floating tiles. The fixed tiles are held in place vertically and radially by four pins that run through the entire tile horizontally. Horizontally, they are held by a mounting bracket as well as two locating pins. However, the locating pins have a large tolerance and are unlikely to be a real constraint.

Figure 1. Top, side, and isometric views of the fixed CSFW tile.

The images above are simplified models of the tile. These images are not to scale and are used only as similar geometries.  The mounting bracket is screwed into the centerstack using bolts that are behind the tile and are tightened through small holes in the surface of the tiles.

The CSFW floating tiles are similar to the fixed tiles in shape. However, they are freer to translate. The tiles are held radially and vertically by the ends of the pins that hold the fixed tiles in place. Horizontally, they are held by fixed tiles on either side. The fixed and floating tiles are placed in an alternating pattern to allow this mounting method. For both the fixed and floating tiles, the following dimensions, loads, or parameters were used in the ANSYS qualification script.

| Variable Name | Value | Units |
| --- | --- | --- |
| Material | 7 (Thermagard) | None |
| Tile width | 3.4 | inches |
| Tile height | 5.8 | inches |
| Tile thickness | 0.75 | inches |
| T1 | 0.5 | inches |
| T2 | 0.5 | inches |
| T3 | 0.0625 | inches |
| T4 | 0.0625 | inches |
| Heat flux | 0.13 × 106 | W/m2 |
| B across face horizontally | 0 | T/s |
| B across face vertically | 590 | T/s |
| B normal to face | 160 | T/s |
| B field across face horizontally | 2.97 | Tesla |
| B field across face vertically | -0.37 | Tesla |
| B field normal to face | 0.07 | Tesla |
| Halo current density across face horizontally | 0 | A/m2 |
| Halo current density across face vertically | 7.76 × 106 | A/m2 |
| Halo current density normal to face | 2 × 106 | A/m2 |

From Art Brooks Tile Moment Estimator:

| Based on linear scaling of SPARK model results |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Note: Yellow Fields for Inputs |  |  |  |
| Time Constant |  |  |  |  |  |  |  |  |  |
|  | Width(in) |  | 3.4 | in |  |  |  |  |  |
|  | Height(in) |  | 5.8 | in |  |  |  |  |  |
|  | radius | r | 0.063637372 | m | Equivalent radius |  |  |  |  |
|  | diameter | d | 0.127274744 | m |  |  |  |  |  |
|  | thickness | t | 0.01905 | m |  |  |  |  |  |
|  | resitivity | rho | 1.17E-05 | ohm-m | ATJ Graphite |  |  |  |  |
|  | time constant | tau | 2.47E-05 | s |  |  |  |  |  |
| Field Change |  |  |  |  |  |  |  |  |  |
|  | Inductive | dB | 0.59 | T |  |  |  |  |  |
|  | Ramp Time | dt | 0.001 | s | Note: Inductive for dt << tau, Resistive for dt >> tau |  |  |  |  |
|  | Resistive | dBdt | 590 | T/s |  |  |  |  |  |
| Induced current |  |  |  |  |  |  |  |  |  |
|  | Inductive | I_ind | 37546.0494 | amps | Note: Linear Scaling From SPARK Analysis |  |  |  |  |
|  | Resistive | I_res | 973 | amps | Note: Linear Scaling From SPARK Analysis |  |  |  |  |
|  | I_ind*EXP(-dt/tau) |  | 0 | amps |  |  |  |  |  |
|  | I_res*(1-EXP(-dt/tau)) |  | 973 | amps |  |  |  |  |  |
| Forces and Moments |  |  |  |  |  |  |  |  |  |
|  | Dipole Current | I_dp | 973 | amps |  |  |  |  |  |
|  | Dipole area | a | 0.012722555 | m2 |  |  |  |  |  |
|  | Dipole Moment | m | 12.37371058 | amps-m2 | Over estimate for resistive solution, reasonable for inductive |  |  |  |  |
|  |  |  | 2888.56443 |  |  |  |  |  |  |
|  | Field | Btf | 2.97 | T |  |  |  |  |  |
|  |  | Bpf | 0.37 | T |  |  |  |  |  |
|  | Torque = mxB | Mpol | 36.75 | N-m | 325 | in-lbs | 27 | ft-lbs |  |
|  |  | Mtor | 4.58 | N-m | 41 | in-lbs | 3 | ft-lbs |  |
| Dipole moment is normal to tile Torques are about poloidal and toroidal axes thru tile |  |  |  |  |  |  |  |  |  |

The central sleeve of the casing is 89 inches long and 23 inches in diameter.  The number of tiles on this section of the casing is 89*23.29*3.1416/3.4/5.8 = 330 tiles. The net moment acting on the thin part of the casing is 27*330*12 = 106,920.  The moment will be reacted by the lower flanges and the upper and lower bellows. The moment will be split between the upper and lower ends of the shell. The shear stress is then 106,920 in-lbs/23.29/pi/.25/2 = 2922.6 psi  (20.15 MPa) shear. Tresca stress is 40.3 MPa. At the top and bottom flanges, torques from all 700 tiles must be reacted. Assuming that all the tiles are similar to the CSFW tiles, the total moment is 27*700*12 = 226,800 in-lbs. This is reacted in the heavier cylindrical sections at each end of the casing. These are 30 inches in diameter and 0.438 inches thick.  The torsional shear is then 226800/2/30/pi/.438 = 2747 psi = 19 MPa Shear or 38 MPa Tresca.

13.0 PF Loading Results

13.1 Stress Results from Ref [2]

- The 2D model identifies EQ31 (PF_Currents_Forces) as producing the max vertical tensile stress in the structure, as PF1a/b upper and PF1a/b lower pull away from the mid-plane with 56 kip.

- In this top-half symmetry model, 12.7 and 43.3 kip are applied to the PF1a and PF1b upper flanges, respectively.

Notice that the max stress of 45 MPa also appears in the center column to transition piece weld, which is comparable to the 50 MPa 2D result (<<300 MPa)

Figure 13.1-1 Stress Results from [2]

13.2 Buckling

L. Myatt did a buckling calculation based on the compressive load from the upper inner PF coils. This showed a large margin. Buckling will be aggravated by the thermal expansion of the central region of the casing, and possibly any tolerance or other geometric imperfections introduced during manufacture, assembly or operation.

Figure 13.2-1 L. Myatt's buckling calculations

L. Myatt's calculations are based on Roark handbook calculations and do not include lateral loading from magnetic misalignments or differential thermal expansion. An Eigenvalue buckling analysis of the centerstack casing was performed. The thermal expansion of the central region was approximated by selecting +/- 1 meter from the equatorial plane and applying 200 degrees C.   The ANSYS instructions say that you should do a static solution first. This was done with the thermal plus Lorentz  loading.

Figure 13.2-2 Buckling Results with the Thermal Loading as a Part of the Load Vector

When ANSYS reports the load factors they are based on the full load vector in the initial static analysis. The results for the casing showed only load factors of 2 when the Euler buckling hand calculations showed factors of 50.

Figure 13.2-3 Eigenvalue Buckling modes and Load Factors with No Thermal Distortions

The Eigenvalue buckling was rerun without the thermal loading and the load factors went up to 160 to 170.

Figure 13.2-4 Eigenvalue Buckling modes and Load Factors with Thermal geometric Distortions

The thermal distortions were input with an initial geometric distortion so that the thermal effects would be considered a geometric imperfection rather than a part of the load. The load factors reduced but not by a substantial amount. The first mode factor went from 159 down to 158.

Figure 13.2-5 Non-Linear Geometry Solution - Radial Displacement as a Function of Load

The NLGEO results show factors of 141.  The NLGEO analysis kept the thermal loads static and the Lorentz loads were stepped up.

Figure 13.2-6  NLGEO  Nominal Geometry, Thermal Expansion Displacement Applied, but not Scaled, Additional Lower Half Oval by 1mm (Representing Fabrication Tolerance)

14.0 Torsional Loading from TF OOP loads

There is a torque from the TF out-of plane (OOP) loading that is transmitted through the bolted and welded connections. This has to be derived from the global modeling of the tokamak. The torsional shear stress in the casing is 6.4 MPa . Based on uniform shear flow, this would be a torque of 6.4e6/6895*.25*22.29*2*pi = 32499 in-lb.

15.0  Seismic Loading Results

More detailed seismic analysis may be found in Reference [12]. In ref [12] , both response spectra and static analyses were used. The results are approximately equivalent in terms of the magnitude of stresses.

Figure 15.0-1 Casing Results for 0.5g lateral static accelerations, Global model run #32

Figure 15.0-1 shows the stresses for a later global model analysis, run#32, with a static lateral acceleration of 0.5 g's applied.  The centerstack casing seismic stresses are small.

Figure 15.0-2 Skirt Results for 0.5g lateral static accelerations, Global model run #32

16.0 Lower Skirt Normal Operating Stress

The skirt is a bolted assembly. It is included in the global model in a fully merged approximate manner. The stress levels are not large and it is assumed that the local details of the skirt will not add significantly to the stresses found in the global model. Figure 16.0-1 shows the general arrangement. Figure 16.0-2 shows the treatment of the skirt in the global model.

Figure 16.0-1 Views of the Lower Skirt and Penetrations for Services

Figure 16.0-2 Views of the Lower Skirt and Penetrations for Services

Figure 16.0-3 Lower Skirt Modeling in the Global Model

Thicknesses and flange details are represented in the global model. This design represents a major improvement in the original design of the centerstack casing support. Initially, support was via three legs which experienced excessive bending. These were replaced with the skirt assembly which had a substantially increased load carrying capability.

Figure 16.0-4 Lower Skirt EQ79 Stress from in the Global Model Run#34

Figure 16.0-5 Lower Skirt Stress from the Global Model Run#32 for 96 no-plasma equilibria and about half off the with-plasma equilibria

17.0  Lower Casing and Skirt Bolt Stress

The lower casing support elements are loaded by the net loads from the PF Lorentz forces and occasionally from disruption halo loads imposed on the casing. Bellows loads from the expansion of the casing put compressive loads on the lower structures. This section is mainly concerned with tensile loads on the bolts and welds.  The bellows compression offsets the bolt and weld tension and is conservatively ignored. There is a torque from the TF out-of plane (OOP) loading that is transmitted through the bolted and welded connections. This has to be derived from the global modeling of the tokamak and is discussed in section 14 of this calculation. The torsional shear stress in the casing is 6.4 MPa. Based on uniform shear flow, this would be a torque of 6.4e6/6895*.25*22.29*2*pi = 32499 in-lb.

Figure 17.0-1 Three Bolt Circles are of Interest

Figure 17.0-2 Another Attempt to Trace the Load Path of the Casing Supports

Figure 17.1-1 Lower PF Mandrel Assembly

17.1 Upper Pf1bl Mandrel Bolting and Weld

The casing flange or divertor flange sits on the PF1b mandrel. The flange and mandrel are welded together and that weld is the primary load carrying element that supports the casing loads. As of August 16, 2011, this weld hadn't been detailed.  Additionally, there are studs that connect across the outside of the mandrel that provide a redundant load path. Spacers were added between flanges connected by the studs and the studs will not load in compression. The studs act to reduce mandrel flange motion under the case loads, and share loads with the welds.

PF Mandrel Assembly Bolting

In the following spreadsheet calculation, two calculations are presented. The first assumed the studs take the moment and tensile loads. The second assumes the weld takes the loads. The weld is the stronger and stiffer of the two load paths and will take most of the loading. The studs and spacers are mainly intended to minimize flexure of the mandrel that might load the coil.

Figure 17.1-1 Upper PF Mandrel Flange and Weld Calculations

The weld stresses are low.  The load path for the vertical tensile loads that result from the moment and Design Point Lorentz loads is more direct than the studs and spacers. The stud loads are overestimated in this calculation. The weld will also take the TF OOP torsional shear stress. This will be less than the 6.4 MPa (928 psi) discussed in section 14 because the PF1b Mandrel diameter is less than the central region of the casing for which the stress was quoted, and the weld thickness (1/4 inch) is the same as the casing wall thickness. Weld stresses are well below the 14 ksi allowable discussed in Figure 5.4-1.

17.2 Pf1bl Mandrel Lower Flange Bolting

Figure 17.2-1 Lower PF Mandrel Assembly

PF1b mandrel is shown in figure 5.5-5 and 17.2-1. The blind hole studs are used to connect the PF1b mandrel into the PF1a flange, which in turn is connected to the skirt flange.  Below, the PF1b mandrel bolting is evaluated as though there are only 23 bolts. Doubling this is recommended. The female thread shear stress is evaluated using the Federal Screw standards [22] for high strength bolting in a lower strength female thread. The Sm allowable for 316 SST is 26.7 ksi (Table 5.4-2).

From [8] 2.4.1.4.2 Special Stress Limits: The average primary shear stress across a section loaded under design conditions in pure shear (e.g., keys, shear rings, screw threads) shall be limited to 0.6 Sm.

Figure 17.2-2 Lower PF Mandrel Bolt Pattern Calculations

In shear, the allowable is 0.6*Sm or 16 ksi. The calculated shear stress is 20 ksi. The tensile stress is 39,031 psi, below the allowable of 62.5 ksi for the high strength bolts. Because of the thread shear, more bolts are needed. The shear capacity of the threads would actually be degraded if the welds were used. It is recommended that Locktite be used instead.

The following is an email exchange between Lew Morris and Peter Titus supporting the need for additional bolts:

Peter,

I've gone ahead and added additonal studs in the available locations on the Upper and Lower PF-1B Winding Mandrels. The number of studs have increased from 21 to 34 on the Lower Mandrel and from 22 to 36 on the Upper Mandrel. Do you need the drawings for specific locations of the studs?

In lieu of Loc-tite on the threads, Jim would like to incorporate a tack weld. This should have no effect on the integrity of the High-strength bolts/studs. Do you agree?

Thanks. Lew

I calculated 20,800 shear for the threads in the lower PF1b flange. I must have mis-counted but I had 23 studs. The allowable for the 316 flange is 16 ksi. The shear area was based on a strong bolt in a soft hole.  I have been recommending ASTM A193 B8M class 2 bolts which are a work hardened 304 bolt, so with corrections on bolt numbers, the shear would be 20,800*23/34 = 14 ksi;  so 34 bolts is OK.  But if the weld softens the stud, I lose on the shear area.  The fed screw fasteners standard allows 0.75 of the hole shear area for strong studs in soft holes vs. 0.5 for soft studs in soft holes. If the whole stud was annealed, stresses would not be acceptable. A tack weld shouldn't anneal much of the stud but I am not sure. I have seen tack welds on nuts outside the stressed thread region, but never near a stressed region of a threaded fastener.  I would definitely prefer Locktite. Also at installation, the studs need to be preloaded to improve the moment carrying capacity.  With the 34 screws, the studs should be preloaded to 3000 lbs. Peter

17.3 Upper Skirt Bolt (section of a ) Circle

Figure 17.3-2 Section Modulus Calculations for the Upper Skirt Bolt Circle

The ASTM A193 B8M Class 2, 5/8-inch bolts would have a stress allowable of the lesser of 125/2 or 2/3*100 = 62.5 ksi, so they are just acceptable for this bolt pattern. The bolts should be preloaded to 75% yield, and this increases the effective section modulus of the bolt pattern. The preloaded bolt friction capacity was calculated based on a friction factor of 0.3 and there is a factor of safety of 63,025/28,100 =2.24 against slippage due to the lateral halo loading.

17.4 Lower Skirt Bolt (section of a ) Circle

Figure 17.4-1 Layout of the Lower Skirt Flange

The skirt bolting is missing a 60-degree section corresponding to the 40-degree opening provided for the OH coax connection. In the initial set of reaction forces provided by Art Brooks, the halo loads were applied at the top of the skirt and had to be translated to the lower bolted flange section which is 22.096 inches below the upper flange. In the latest transmittal of loads (Appendix B, ref [19] ) the loads are provided at the base of the skirt.  In this transmittal, the lateral load is 160,000N or 36,000 lbs. The lower bolt pattern of the skirt is different than the upper pattern. There are 11, 5/8-inch bolts in the lower circle.

ASTM A193 B8M class 2  bolts with 100 ksi yield are recommended. The bolts should be pre-tensioned to 75% yield. 5/8-inch bolts have a stress area of .2256 in^2 so this would be a load of 16,920 lbs each. Based on a friction factor of 0.3, 11 bolts would have a shear capacity of 11*.3*16,920 = 55,836 lbs, well in excess of the 37,092 lbs applied shear.

So far, the peak halo vertical tensile forces have been considered to act concurrently with the peak halo moment. A review of Art Brooks plots (Appendix B) shows that the peak moment occurs after the peak vertical load.

Figure 17.4-2 Section Modulus of the Lower Skirt Flange Bolting

Figure 17.4-3 Lower Skirt Flange Bolt Stress Calculations

17.5 Bolt Loads on the Lower TF Flag Keys

The G-10 ring on top of the TF flags in the bottom of the machine is attached using countersunk bolts and thread inserts to the TF flags. The centerstack skirt and the OH bottom cage flange are, in turn, attached to the G-10 using bolts and thread inserts into the G-10 ring at a more radially-outward bolt pattern.  Figure 17.5-1 and 2 show this interface.

Figure 17.5-1

Figure 17.5-2

The OH Coax Box is analyzed in ref [23]. The OH rests on a "cage" that also is bolted into the stacked flanges above the TF flag keys. The OH is not loaded laterally. Ref [20] considers the net vertical Design Point combined OH and Inner PF loading. Additional loads from the lateral and vertical halo loading on the casing must be considered in ref [20]. The stack up of flanges, the structure around the coax, and the OH support cage are assumed to bridge the open section of the skirt.

Figure 17.5-3 Lower Flag Key Loading

The bolt load at the lower flag key is calculated to be 9984 lbs without taking credit for the preload moment capacity. This is less than the 12000 lbs considered in ref [20].

Appendix A - Email Data

From A Brooks Sept 19 2011

Peter,

The latest dynamic reaction loads to the support are significantly different from what you have (see the updated calculation of Halo Currents I sent you last week). The peak reaction loads are (from figures 23 and 24 for Fast Quench):

Fx (Lateral force)                              150,000 N

Fz (Vertical force)                            110,000 N

My (Moment about base)             90,000 N-m

The vertical force appears to come from a Poisson effect with the large radial forces and resulting hoop stresses in the mid section of the CS. This produces large Sz (vertical stress) which reacts at the base during the dynamic response.

The lateral force is still lower than the 50,000 lbs (222,000 N) Mark was using.

Art

Peter,

Comments on the Centerstack Casing Stress Summary Calc:

1)     Peak stresses in CS from Halo are now reported to be 43.7MPa, down from previous 59.6 MPa

2)     Peak Thermal Stress has dropped to 192 MPa from 280 MPa

Art

Attached is an update to the forces and moments on the CS Base Support and Bellows. The net applied load is 250 kN, up from the 140 kN previously reported. This is due to the guidance from Stefan to adjust the TPF at the strike point such that the TPF at the midplane is 1.35 ( the strike point needed to be increased to 1.60). The 250 kN is very close to the estimated value obtained by assuming a constant TPF of 1.35 over the +/- 0.6m height.

The peak moment is now 130 kN-m and the peak reaction load is 250 kN at the base support. There is also a sizeable reaction at the bellows/bumper, 220 kN, though it is not in phase with the reaction load at the base (see last two figures in attached).

Art

Thu 3/11/2010 8:21 AM

Peter,

Summing up the applied halo forces for the resistive distribution scenario (for the strike at z=+/-0.6m) with PF and TF (1/R) fields I get:

Applied Load Sum on CS

Fx = -30695.6 N, Fy=Fz=0

Mx =  80400.7 N-m, My=Mz=0

I ran these thru a stress pass constraining all the points on the top and bottom flanges and looked at the reaction loads:

Reaction Loads on CS when Upper & Lower Flanges Fully Constrained

Fx, N       Fy          Fz          Mx, N-m           My          Mz

Up    15347.      32464.      44662.      -40200.9    56846.7    -201.8

Low   15349.     -32463.     -44661.      -40199.6    -56848.9    201.8

The sum of the Up and Low values do add to negative the applied loads as expected. It just highlights the need to look at the reaction moments as well when considering support design loads.

Art

Appendix B

Halo reaction loads for the base skirt with the compliance of the G-10 flange modeled.

Peter,							Dec 19 2011

I've extracted the forces and moments at the interface of the base of the CS and top of the lower support. The peak vertical load is lower (~60 kN) than at the bottom of the lower support (~80 kN). The moments are about the same (~95 kN-m) but occur at different times. The numbers are extracted using fsum on the interface nodes with the lower support elements and are the total force (static + inertial + damping).

Art

In response to a request for the load at the PF1b mandrel elevation:

Peter,							Dec 22 2011

The lateral load at that elevation is ~125 kN. The plots I sent contain the transient behavior of each of the three load and moment directions.

Art

Peter,

Adding the compliant G10 plate and structure sitting on the TF flags has reduced the moment (now measure at the G10, z=-2.7m) to a peak of 95 kN-m during the dynamic response. The net lateral force has dropped to 160 kN. The bellows/bumper reaction drop slightly to 200 kN and again is not in phase with the reaction load at the base (see attached plots).

Art

---------- Forwarded message ----------
From: Arthur Brooks <abrooks@pppl.gov>
Date: Tue, Nov 1, 2011 at 12:31 PM
Subject: Halo Reaction Forces with Bumper
To: Peter Titus <ptitus@pppl.gov>

Peter,